5.8G 450M 3KM CPE Digital Bridge

▼Overview

TBC-APN2621AB is a 802.11A/N/AN 450Mbps outdoor CPE that support 5G full band. With LED digital display, it’s easy to complete point to (multiple)point matching without computer. Support 24V POE (up to 80m according to cable material) & 12V 1A DC power supply. It’s designed for video & data transmission in short and medium distance.

▼Hardware Features

- 450Mbps High Speed & NAT High Conversion Rate

- TBC-APN2621AB comply with 802.11A/N/AN technology to provide up to 450Mbps wireless access speed，and high NAT conversion rate of over 93% to achieve faster download/upload speed, making online surfing faster.

- Efficient Energy Saving

- Support IEEE802.3az. When there is no addressable frame,the transceiver will enter low power consume mode, and if new frame comes, it will back to active mode instantly. According to the actual flow of the port, adjust energy consumption automatically, and switch fast between full speed & low consume mode,which save 30% power cost.

- Beamforming Technology

- Beamforming Technology achieves superior Wi-Fi bandwidth utilization and increases Wi-Fi range by creating efficient, highly targeted Wi-Fi connections.

- Simple & Efficient Pairing

- Need not to be professional nor have a computer, matching the host & subordinative bridge  just by setting the Nixie tube the same number （within 8 points）.

- Support 5G Full Band

- Support 36、40、44、48、52、56、60、64、149、153、157、161、182、186、190、194 band. The special band can be opened if need.

- ▼产品参数

| Hardware Configuration |  |
| --- | --- |
| Master Chip | MTK7628, 8MB DDR RAM |
| Flash | 2MB |
| Size | 260*90*37mm |
| Wireless Technology | 5.8G（802.11a/n/an）1T1R |
| Antenna | Inbuilt 14dBi Dual Polarization HG Directional Panel Antenna |
| Wireless Speed | 5.8g 450Mbps |
| Network Port | 2*10/100Mbps RJ45 |
| Power | DC 12V 1A / POE 24V 1A |
| Button | 1*Digital Switch, Short Press to Change the Displayed Number 1*Reset，Long Press 5s to Reset 1*Toggle Switch, Dial to Convert Host/Subordinative |
| Transmission Distance | 1KM |
| Operating Frequency | 802.11a/n/an : 5.1GHz~5.8GHz (China) |
| Output | 11a @54M:25±2DB, @6M:27±2DB 11n @MCS7:23±2DB, @MCS0:25±2DB |
| EVM | 11n MCS; Less than（dB）. Modulation; Coding rate. BPSK; 1/2; -5. |
| EVM | 5GHz; Symbol; Parameter; Rx(dBm). 802.11a; Srf (OFDM); 1Rx chain; 6Mbps; -88. 54Mbps; -72. MCS7; -64.5. MCS9; -57.5. 802.11ac VHT80; 1 Stream(1x1); MCS0; -79. MCS7; -62. MCS9; -55.5. 2 Stream(2x2); MCS0; -78. MCS7; -61. MCS9; -54.5. |
| Support Wireless Speed | 802.11an: 6.5Mbps-867Mbps 802.11n: 6.5Mbps and 300Mbps |
| Work Environment | TEMP：-30℃～+55℃（work），-40℃ ～+70℃（storage） Humidity(Non-Condensing)：10％～90％（work），5％～95％（storage） |
| Waterproof Rate | IP65 |
| Signal Indicator LED | - Bridge Access Point：Output Indicator - Under 25%（SIG1 on），25%~50%（SIG1-SIG2 on），50%~75%（SIG1-SIG3 on），75%~100%（SIG1-SIG4 on） - Bridge Client：Signal Strength Indicator The light on in turn when connection fails. If connection succeed, - 0~-65dBm（SIG1-SIG4 on），-66~-75dBm（SIG1-SIG3 on），-76~-85dBm（SIG1-SIG2 on），under -86dBm（SIG1 on） |
| LAN1 GigE Indicator | Flicker When Wire Data Transmission , Off When Wire Disconnects |
| Wireless Indicator | Flickers When Bridge is Running Normally with Power |
| Power Indicator | With Power, On Without Power, Off |